|  | LEARNING UNIT — DRAFT Judging a hard drive by its own records |
| --- | --- |

# Reading a drive's SMART results

A drive keeps its own health records. SMART — Self-Monitoring, Analysis and Reporting Technology — is the part of a drive's firmware that counts its errors, tracks its age and temperature, and runs self-tests on request. The operating system does not grade the drive; the drive grades itself, and smartctl is how you ask to see the marking.

This unit covers reading those records: the one command that shows everything, the five numbers that decide whether a drive can be trusted, and the two traps that make healthy drives look dying and dying drives look healthy.

NOTE

A self-test runs inside the drive, not on the computer. The terminal that started it will never print a result, and closing that terminal changes nothing. The result waits in the drive's own log until something asks for it.

# The command

sudo smartctl -H -A -l selftest /dev/sda

| Part | What it does |
| --- | --- |
| sudo | Drive-level queries need administrator rights. |
| smartctl | The control tool from the smartmontools package. |
| -H | The drive's overall verdict on itself — one line, PASSED or FAILED. |
| -A | The attribute table: the drive's running counters. |
| -l selftest | The self-test log, including the test you started. |
| /dev/sda | Which drive to ask. |

TIP

Device names are handed out at boot and can swap between boots: the drive called sda today can be sdb tomorrow. Before acting on any result, confirm you are reading the drive you think you are: sudo smartctl -i /dev/sda prints the model and serial number, which match the label on the physical drive.

Prefer a window? Ubuntu ships Disks (search the applications grid for "Disks"). Select the drive, open the menu in the title bar, and choose SMART Data & Self-Tests. Same numbers, same log, and it can start and stop tests too.

# Three layers, three different weights

smartctl answers at three levels, and they are not equally trustworthy.

- The overall verdict (-H) is the weakest. PASSED means only that no counter has yet crossed its failure threshold — drives routinely report PASSED while visibly deteriorating. Treat it as "not already condemned", never as "healthy".

- The self-test log (-l selftest) is the exam result. A long test reads every sector on the platter, so "completed without error" means the whole surface has actually been read, recently.

- The attribute table (-A) is the continuous record. It is where deterioration shows first, and the trend across weeks matters more than any single reading.

# Reading the self-test log

The log lists recent tests, newest first:

Num Test Status Remaining LifeTime(hours) # 1 Extended Completed without error 00% 10831 # 2 Short Completed without error 00% 9905

| Status | Meaning | What to do |
| --- | --- | --- |
| Completed without error | Every sector read cleanly. | The pass you wanted. |
| Completed: read failure | Bad media found; the log records where. | Treat the drive as failing — see the verdicts below. |
| Self-test routine in progress | Still running; the percentage counts down to 00%. | Wait. Nothing else. |
| Interrupted (host reset) | A reboot or suspend cut it short. | No verdict either way. Run it again. |
| Aborted by host | Something cancelled it. | Run it again. |

CHECK

LifeTime(hours) is the drive's power-on-hours count at the moment that test ran. Compare it against Power_On_Hours in the attribute table: if the two match to within a few hours, the top entry is the test you just ran, not a relic from years ago.

# The five numbers that decide

Backblaze, which operates hundreds of thousands of drives and publishes its failure statistics, tracks five SMART attributes as failure predictors: in its data, 76.7% of failed drives showed a non-zero value in at least one of these five before failing. Read the RAW_VALUE column for all of them.

| ID | Attribute | What it counts |
| --- | --- | --- |
| 5 | Reallocated_Sector_Ct | Sectors that failed and were quietly replaced from the drive's spare pool. |
| 187 | Reported_Uncorrect | Errors the drive had to report back to the computer as uncorrectable. |
| 188 | Command_Timeout | Operations that took so long the drive gave up on them. |
| 197 | Current_Pending_Sector | Sectors failing to read right now, awaiting a verdict. |
| 198 | Offline_Uncorrectable | Sectors that could not be read even during the drive's own scans. |

A healthy drive shows zero for all five. The lifecycle behind the two sector counts: a sector that fails a read goes pending (197). If a later read succeeds, it leaves the list; if the spot is confirmed bad, the drive swaps in a spare and the count moves to reallocated (5). Reallocation is the drive consuming its finite stock of spares — the mechanism working as designed, but the need for it is the warning.

WARNING

The Seagate panic trap. On Seagate drives, Raw_Read_Error_Rate (1), Seek_Error_Rate (7) and Hardware_ECC_Recovered (195) show raw values in the millions or billions. These are not errors: Seagate packs the total count of operations into the low bits of the same raw number, so it grows enormous on a perfectly healthy drive while the actual error count — the high bits — stays at zero. Judge those three by their normalised VALUE holding steady against THRESH, or ignore them. Only the five attributes above are read raw.

TIP

UDMA_CRC_Error_Count (199) counts data corrupted on the way between the drive and the motherboard. A rising 199 with the five above at zero is a cable fault, not a drive fault: reseat or replace the SATA data cable. The count never resets, so what matters is whether it stops rising.

# The other columns, briefly

Each attribute row also carries VALUE, WORST and THRESH — a normalised score that starts high (typically 100 or 200), falls as the drive degrades, and fails when it meets THRESH; WHEN_FAILED says whether that has ever happened. These columns are what the -H verdict is built from, which is exactly why -H is slow to alarm: for the five counters above, the raw value moves long before the normalised score follows.

Two attributes are context rather than verdicts. Power_On_Hours (9) is the drive's total working life, useful for judging how much a clean record actually covers — a drive that has spent most of its years switched off has had few chances to discover its own bad sectors, which is precisely what a long test corrects. Temperature_Celsius (194) should sit comfortably below about 40 °C in steady use.

# What the results decide

| Result | Verdict |
| --- | --- |
| Long test completed without error, all five counters at zero | Fit for its role. The surface has actually been read, end to end, recently. |
| Test passes, but a counter is small and non-zero | Usable, not trustworthy: nothing irreplaceable lives on it. Re-test in a few weeks — growth is the signal, not the absolute number. |
| Read failure, or counters climbing between checks | Plan its retirement. Copy anything wanted off it now, while it still reads, starting with what matters most. |

NOTE

One clean pass is evidence, not immunity. Backblaze's same figure read the other way: roughly a quarter of its failed drives showed no warning in any of the five attributes. A backup schedule, not a test result, is what protects data — the test says whether a drive is fit to hold data today; the backup is for the drive that fails without notice tomorrow.

# Keeping watch without doing this by hand

The smartmontools package includes smartd, a background service that polls every drive on a schedule, runs periodic self-tests, and raises an alert when a counter moves. Its configuration lives in /etc/smartd.conf. Setting it up belongs with monitoring and alerting, in a later unit — the point here is that the manual check this unit teaches is also available as a standing guard.

Re-run a long test before trusting a drive with a new role, after the machine is physically moved, and on something like a yearly rhythm in between. The Disks application shows a one-line assessment for every drive whenever the quick answer is enough.

Sources: Backblaze, "What SMART Stats Tell Us About Hard Drives" (backblaze.com/blog/hard-drive-smart-stats), which names the five attributes and the 76.7% figure; smartmontools documentation (smartctl(8), smartmontools.org) for flag behaviour and self-test statuses; the Seagate raw-value encoding is community-documented (smartmontools issue tracker, TrueNAS forums) rather than published by Seagate, and is stated here with that weight. Checked 2026-08-09.